Assignment 1 : Analyze packet capture and analysis

Name : Atish Kadam

Roll Number : CS25MTECH14003

PART-A: Analyze packet trace of your browsing session

1. Which of the following protocols are shown as appearing (i.e., are listed in the

Wireshark “protocol” column) in your trace file: TCP, QUIC, HTTP, DNS, UDP,

TLSv1.2, etc?

Ans-

TCP QUIC HTTP MDNS DNS

2. How long did it take from when the HTTP GET message was sent until the

HTTP OK reply was received? (By default, the value of the Time column in

the packet-listing window is the amount of time, in seconds, since

Wireshark tracing began. (If you want to display the Time field in time-of-

day format, select the Wireshark View pull down menu, then select Time

Display Format, then select Time-of-day.)

Time= 8.989596544 - 8.579439083 = 0.410157461

3. What is the Internet address of the gaia.cs.umass.edu (also known as

www-net.cs.umass.edu)? What is the Internet address of your computer

or (if you are using the trace file) the computer that sent the HTTP GET

message?

To answer the following two questions, you’ll need to select the TCP packet containing

the HTTP GET request. The purpose of these next two questions is to familiarize you

with using Wireshark’s “Details of selected packet window”; see Figure 3. To answer

the first question below, then look in the “Details of selected packet” window toggle the

triangle for HTTP (your screen should then look similar to Figure 5); for the second

question below, you’ll need to expand the information on the Transmission Control

Protocol (TCP) part of this packet.

Ans  - 
While Sending GET-

Source Address : 192.168.0.103  (My Computer)

Target Address : 128.119.245.12 (Server Address)

While receiving OK -

Source Address : 128.119.245.12 (My Computer)

Target Address : 192.168.0.103 (My Computer)

4. Expand the information on the HTTP message in the Wireshark “Details of

selected packet” window (see Figure 3 above) so you can see the fields in the

HTTP GET request message. What type of Web browser issued the HTTP

request? The answer is shown at the right end of the information following the

“User-Agent:” field in the expanded HTTP message display. [This field value in

the HTTP message is how a web server learns what type of browser you are

using.]

● Firefox, Safari, Microsoft Internet Edge, Other

Ans-

What type of Web browser issued the HTTP Request?

Mozilla Firefox

Mozilla/5.0 (X11; Ubuntu; Linux x86_64; rv:142.0)

5. Expand the information on the Transmission Control Protocol for this packet in

the Wireshark “Details of selected packet” window (see Figure 3 in the lab

writeup) so you can see the fields in the TCP segment carrying the HTTP

message. What is the destination port number (the number following “Dest Port:”

for the TCP segment containing the HTTP request) to which this HTTP request is

being sent?

Source Port : 57986

Destination Port : 80

6. Print the two HTTP messages (GET and OK) referred to in question 2

above. To do so, select Print from the Wireshark File command menu, and

select the “Selected Packet Only” and “Print as displayed” radial buttons,

and then click OK.

GET -

OK -

7. Answer Q1-Q3 again by visiting the following links from your web browser.

Save each visit as a separate pcap file.

o https://www.icann.org/

Protocols used -  TLSv1.2,TLSv1.3,TCP

(eth,tls,frame,tcp,ip) – copied from statistics->protocol hierarchy

Time -

- We dont get any HTTP messages (GET and OK)

- The Browser (Mozilla Firefox) first sends a TLS handshake to the ICANN server to establish a secure connection.

- Once the connection is established

- We dont see the HTTP message on wireshark as it is hidden in TLS messages

Calculating Time between Client hello and Server hello

4.137232741 (Server Hello)- 4.048919126 (Client Hello)= 0.088313615 seconds

Time taken by the server to respond to clients initial secure connection request.

While Client hello ->

Source IP (My PC): 192.168.0.103

Destination IP (ICANN server): 104.18.3.93

While Server hello ->

Source IP (ICANN server): 104.18.3.93

Destination IP (My PC): 192.168.0.103

o http://example.com

Protocols used -  (Ipv6,eth,icmpv6,udp,tls,igmp,data,frame,arp,http,data-text-lines,quic,tcp,dns,ip) – copied from statistics->protocol hierarchy

Time -  7.021521143 - 6.671541083 = ~0.34998 seconds

While Client hello ->

Source IP (My PC): 192.168.0.103

Destination IP (ICANN server): 23.215.0.136

While Server hello ->

Source IP (ICANN server): 23.215.0.136

Destination IP (My PC): 192.168.0.103

o www.youtube.com

Protocols used -  (Ipv6,eth,udp,tls,data,mdns,frame,arp,http,data-text-lines,quic,tcp,dns,icmp,ip) – copied from statistics->protocol hierarchy

Time -

- We dont get any HTTP messages (GET and OK)

- The Browser (Mozilla Firefox) first sends a TLS handshake to the ICANN server to establish a secure connection.

- Once the connection is established

- We dont see the HTTP message on wireshark as it is hidden in TLS messages

Calculating Time between Client hello and Server hello

4.446278253 - 4.436960385= 0.009317868 seconds

Time taken by the server to respond to clients initial secure connection request.

While Client hello ->

Source IP (My PC): 192.168.0.103

Destination IP (You tube server): 199.232.105.91

While Server hello ->

Source IP (You tube server): 199.232.105.91

Destination IP (My PC): 192.168.0.103

8. Compare and contrast what you observed in Wireshark or in your browser

when you visited the above four websites. Does your output vary when

revisiting the same site after some time? Why or why not?

9. Wireshark also helps to gather a lot of stats about the flows. Refer to this

help page for more details:

https://www.wireshark.org/docs/wsug_html_chunked/ChStatistics.html

Your task is to take one of the pcaps that you collected when you visited

different websites and present some interesting stats like packet lengths,

Flow Graphs, TCP Throughput, etc. You can insert screenshots into your

assignment report and briefly comment on key takeaways.

General statistics

Ans -

Capture File Properties about the capture file.

Protocol Hierarchy of the captured packets.

Conversations e.g., traffic between specific IP addresses.

Endpoints e.g., traffic to and from IP addresses.

I/O Graphs visualizing the number of packets (or similar) in time.

The “Capture File Properties” Dialog

The “Protocol Hierarchy” Window

The “Conversations” Window

The “Endpoints” Window

Packet Lengths

The “I/O Graphs” Window

PART-B: Analyze packet trace of traceroute session

Task-1: Capture traceroute traffic to/from one of the websites visited as part of

PART-A using wireshark and answer the following in a google doc. Feel free to

include screenshots from terminal/wireshark to support your answers. [8 Marks]

1. What protocol among ICMP/TCP/UDP is used to send probe packets?

Identity key fields and comment on their values.

Ans -

UDP - It is used to send probe packets

2. Can you change the default protocol used to send probes? Demonstrate it

and comment on how the last probe response differs among

ICMP/TCP/UDP.

Sol -

For UDP packets

When i run traceroute example.com on terminal

For ICMP-

When i run traceroute -I example.com on terminal

For TCP

When i run traceroute -T example.com on terminal

3. What is the typical gap (delay) between probe packets?

Ans -

For TTL=1

Packet -70

Packet - 71

Packet - 72

(Packet 71 -Packet 70)=17.450546309-17.450498782=0.000047527

(Packet 72 -Packet 71)=17.450566739-17.450546309=0.000020430

4. What is contained in probe responses? What protocol is used in probe

responses when ICMP/TCP/UDP are used to send probes?

Ans-

Intermediate Hops - ICMP Time Exceeded

Destination - ICMP Echo Reply

Similarly for TCP -

Intermediate hops- ICMP Time Exceeded

Destination - Either TCP SYN/ACK

Similarly for UDP

Intermediate Hops - ICMP Time

Destination - UDP (High numbered Ports) and No response

5. Which protocol has TTL field and comment on how the values of this field

varied across probes and responses?

IPV4 Protocol has TTL Field.For each Probe value of TTL increased by One for Each Hop.

6. How long did it take to get the output of the traceroute session? Which is

the bottleneck router?

ICMP BottleNeck Router
49.45.4.85

Corresponding Jump - 
337.50 Seconds

7. Do you see any stars (*) in the output? Discuss the potential reasons

behind the presence of these stars in the output.

- There are stars (*) in the traceroute data.

- These show up when a router fails to react to at least one of the delivered probes.

- This may occur because the router is too busy, firewalls are blocking ICMP packets, or the router is set up to not respond.

- It's common and doesn't always mean that something is wrong, particularly if the trace goes all the way to the destination.

8. How many probe packets are sent in bursts? What is the inter packet time

difference between consecutive packets in the first burst? How much time

elapsed between the first request and its response?

Ans-

- The terminal output seen above allows us to see that:

- There are three probe packets each burst.

- Based on the difference between the 3 ms and 2 ms readings, the inter-packet time delay between successive packets in the first burst was approximately 1 ms.

- At hop 1, with TTL = 1, the time interval between the initial request and its response was approximately 3 ms.

- Time elapsed between first re	quest and its response is 46.263678-45.983575=0.280103 seconds

Task-2: Answer Task-1 Q.6 and Q.8 using tcpdump (yet another packet

capturing tool, no GUI, CLI only) instead of wireshark to capture traffic to/from

one of the remaining websites visited as part of PART-A. Make use of different

filtering options in tcpdump to get concise output for answering these questions.

[2 Marks]

Ans -

Question - 6

Time for output session = 46.494665- 45.983575=0.51109

Bottle Neck Router - 116.119.44.134

Question - 8

3  Probe packets are sent per burst

To keep the output concise and relevant, I used the following tcpdump options:

Disables DNS lookups (-n)

Captures only N packets (-c N)

Protocol filters (icmp, tcp, udp) : Limits capture to specific traffic types

Shows time differences between packets for response timing analysis (-ttt / -tttt)

For further refinement, I added:

-i any: Captures packets on all available interfaces

-s0: Captures the entire packet (not just headers)

Protocol-specific filters : Focuses on the relevant protocol for better clarity