Chapter 5 Network Layer: Control Plane

------------------------------------------------------------------------------------------------------------------------

- ▪introduction

  - 1. Per-Router Control Plane:
Each router makes its own forwarding and routing decisions independently using its local routing algorithms and information exchanged with neighboring routers (e.g., via OSPF, RIP, BGP).

  - 2. SDN (Software-Defined Networking) Control Plane:
A centralized controller (separate from routers) manages routing and forwarding decisions for the entire network. Routers (or switches) simply follow the instructions from this controller.

- ▪ routing protocols

- Routing protocol goal: determine “good” paths from sending hosts to receiving host, through network of routers

- path: sequence of routers packets traverses from given initial source host to destination host

- “good”: least “cost”, “fastest”, “least congested”

- routing: a “top 10” networking challenge

- Global (Link State) → Full network view, centralized computation per router.

- Decentralized (Distance Vector) → Partial network view, iterative updates with neighbours.

- Static → Fixed routes.

- Dynamic → Adaptive routes

  - ▪ link state

Dijkstra’s Link State Routing algorithm

- centralized: network topology, link costs known to all nodes

  - accomplished via “link state broadcast”

  - all nodes have same info

- computes least cost paths from one node (“source”) to all other nodes

- gives forwarding table for that node

- iterative: after k iterations, know least cost path to k destinations

D(v) = min ( D(v), D(w) + cw,v )

oscillations possible –

when link costs depend on traffic volume, route oscillations possible

  - ▪ distance vector

| Aspect | Link State (LS) | Distance Vector (DV) |
| --- | --- | --- |
| Information Used | Global — all routers know full topology and link costs | Local — routers know only neighbors’ costs |
| Message Complexity | O(n2)messages for n routers | Exchanges only between neighbors; message count and convergence vary |
| Speed of Convergence | O(n2)time; may cause oscillations | Convergence time varies; may have routing loops and count-to-infinity problem |
| Robustness | If a router advertises incorrect link cost, it affects only its own computation | If a router advertises wrong path cost, error propagates through the network (black-holing possible) |
| Computation | Each router computes its own shortest paths (e.g., Dijkstra) | Each router updates distances iteratively (e.g., Bellman-Ford) |
| Example Protocols | OSPF, IS-IS | RIP |

Based on Bellman-Ford (BF) equation (dynamic programming):

Let Dx(y): cost of least-cost path from x to y. Then: Dx(y) = min { cx,v + Dv(y) }

In Distance Vector Routing, every router tells its neighbors the cost to reach all other routers.

✅ Good news travels fast

When a link cost decreases, updates spread quickly.

Example:

x —— y —— z

- Suppose link cost y–x changes from 4 → 1.

- y updates its table and informs z.

- z recalculates its cost to x (now smaller) and updates too.
Since costs are improving, routers quickly accept the update —
that’s why “good news travels fast.”

⚠️ Bad news travels slow (Count-to-Infinity Problem)

When a link cost increases or breaks, routers can mislead each other using old info.

Example:

x —— y —— z

- Originally: y–x = 4, y–z = 1, z–x = 5

- Then y–x becomes 60 (bad link).

- y thinks: “z can reach x in 5, so my cost via z = 1 + 5 = 6.”

- z then thinks: “y can reach x in 6, so my cost = 1 + 6 = 7.”

- y updates again: 8 … then 9 … and so on.

This endless loop of increasing values is the count-to-infinity problem, meaning “bad news travels slow.”

💡 Fixes: Modern routing protocols use
Split Horizon, Poison Reverse, and Hold-Down Timers to stop such loops.

- ▪ intra-ISP routing: OSPF

Making Routing Scalable

- The Internet has billions of destinations:

  - Routers can’t store all destinations in routing tables.

  - Routing table exchanges would overload links.

- Administrative autonomy:

  - The Internet is a network of networks.

  - Each network’s admin may want independent routing control.

2. Internet Approach to Scalable Routing

Autonomous Systems (AS)

- The Internet is divided into regions called Autonomous Systems (AS) (also called domains).

- Each AS is administered independently.

Two Types of Routing

(a) Intra-AS Routing (Intra-Domain)

- Routing within the same AS.

- All routers in an AS must run the same intra-domain routing protocol.

- Routers in different ASes can use different intra-domain protocols.

- Gateway routers:

  - Located at the edge of their AS.

  - Have links to routers in other ASes.

(b) Inter-AS Routing (Inter-Domain)

- Routing between ASes.

- Gateway routers participate in inter-domain routing in addition to intra-domain routing.

3. Interaction Between Intra-AS and Inter-AS Routing

| Aspect | Description |
| --- | --- |
| Forwarding Table Configuration | Determined by both intra-AS and inter-AS routing algorithms. |
| Intra-AS Routing | Determines entries for destinations within the AS. |
| Inter-AS + Intra-AS Routing | Together determine entries for external destinations. |

4. Inter-AS Routing: Role in Intra-Domain Forwarding

5. Intra-AS Routing (Routing Within an AS)

Common Intra-AS Routing Protocols

| Protocol | Type | Key Features |
| --- | --- | --- |
| RIP (Routing Information Protocol) [RFC 1723] | Distance Vector (DV) | - Classic DV protocol- DVs exchanged every 30 seconds- Obsolete (no longer widely used) |
| EIGRP (Enhanced Interior Gateway Routing Protocol) | Distance Vector (DV) | - DV-based, |
| OSPF (Open Shortest Path First) [RFC 2328] | Link-State | - Publicly available ("open")- Uses Dijkstra’s algorithm- Multiple link cost metrics (bandwidth, delay) |

6. OSPF (Open Shortest Path First) Routing

Key Characteristics

- Open: Publicly available protocol.

- Link-State Based:

  - Each router floods link-state advertisements (LSAs) to all other routers in the AS (sent directly over IP, not TCP/UDP).

  - Each router builds a complete network topology map.

  - Uses Dijkstra’s algorithm to compute the shortest paths and fill its forwarding table.

- Multiple Metrics:

  - Can consider bandwidth, delay, etc.

- Security:

  - All OSPF messages are authenticated to prevent malicious tampering.

7. Hierarchical OSPF

Two-Level Hierarchy

- ▪ routing among ISPs: BGP

Interconnected Autonomous Systems (ASes)

2. Internet Inter-AS Routing: Border Gateway Protocol (BGP)

BGP Overview

- BGP (Border Gateway Protocol) is the de facto inter-domain routing protocol of the Internet.

- Often called the “glue that holds the Internet together.”

Functions of BGP

BGP allows each AS to:

- Advertise its existence and the destinations it can reach:

  - “I am here; here’s who I can reach and how.”

- Obtain reachability information from neighboring ASes (via eBGP).

- Determine routes to other networks based on:

  - Reachability info, and

  - Local routing policies.

- Propagate reachability within its AS (via iBGP).

- Advertise reachability to external ASes.

4. BGP Basics

How It Works

- Suppose AS3 gateway 3a advertises path (AS3, X) to AS2 gateway 2c:

  - AS3 promises AS2 it will forward datagrams toward destination X.

- BGP Session:

  - A semi-permanent TCP connection between two BGP peers.

  - Peers exchange BGP messages containing advertisements of network prefixes.

  - BGP is a Path Vector Protocol — each route includes the full AS path.

5. BGP Protocol Messages (Defined in RFC 4371)

| Message Type | Function |
| --- | --- |
| OPEN | Establishes a TCP session and authenticates peers. |
| UPDATE | Advertises new paths or withdraws old ones. |
| KEEPALIVE | Keeps the session active (sent periodically); also acknowledges OPEN. |
| NOTIFICATION | Reports errors and closes the connection. |

6. Path Attributes and BGP Routes

A BGP route = Prefix + Attributes

Key Attributes

- AS-PATH – list of ASes a route advertisement has passed through.

  - Used for loop prevention and policy decisions.

- NEXT-HOP – indicates the specific internal router to use when forwarding to the next AS.

Policy-Based Routing

- Each AS applies import policies:

  - Accept or reject routes (e.g., “never route through AS Y”).

- AS export policies decide whether to advertise routes to other ASes.

  - Enables control over traffic flow and business relationships.

7. BGP Path Advertisement Example

Normal Case

- AS2 router 2c receives path advertisement AS3,X (via eBGP) from AS3 router 3a

- based on AS2 policy, AS2 router 2c accepts path AS3,X, propagates (via iBGP) to all AS2 routers

- based on AS2 policy, AS2 router 2a advertises (via eBGP) path AS2, AS3, X to AS1 router 1c

Multiple Path

gateway router may learn about multiple paths to destination:

- AS1 gateway router 1c learns path AS2,AS3,X from 2a

- AS1 gateway router 1c learns path AS3,X from 3a

- based on policy, AS1 gateway router 1c chooses path AS3,X and advertises path within AS1 via iBGP

10. Hot Potato Routing

- A router may learn multiple gateways to a destination (e.g., 2a and 2c).

- Hot Potato Routing:

  - Choose the gateway with the least intra-domain cost, regardless of inter-domain path length.

  - Example:

    - AS2 router 2d has two gateways to X (via 2a, 2c).

    - Chooses the nearest one (lowest OSPF cost) to reduce internal load.

- Focus: Minimize internal (intra-AS) cost, not total AS hops.

11. Achieving Policy via BGP Advertisements

Example 1 – Provider-Customer Policy

- B gets no “revenue” for routing CBAw, since none of C, A, w are B’s customers

- C does not learn about CBAw path

Example 2 – Dual-Homed Customer Policy

- policy to enforce: x does not want to route from B to C via x

- .. so x will not advertise to B a route to C

12. BGP Route Selection Criteria

When multiple routes to a destination exist, BGP chooses based on:

| Priority | Criterion | Description |
| --- | --- | --- |
| 1 | Local Preference | Highest value preferred; used for policy-based decision. |
| 2 | Shortest AS-PATH | Fewer AS hops preferred. |
| 3 | Closest NEXT-HOP (Hot Potato) | Lowest intra-domain cost to reach next hop. |
| 4 | Other Tiebreakers | e.g., router ID, origin type, etc. |

13. Why Separate Intra-AS and Inter-AS Routing?

| Aspect | Intra-AS Routing | Inter-AS Routing |
| --- | --- | --- |
| Policy | Single administrative control → minimal policy conflicts. | Different admins → must enforce policies (who carries whose traffic). |
| Scale | Within one AS → simpler, smaller tables. | Across Internet → hierarchical routing reduces table size and updates. |
| Performance Focus | Emphasis on optimal paths and low delay. | Emphasis on policy and control rather than raw performance. |

- ▪ SDN control plane

Software Defined Networking (SDN)

1. Traditional Internet Network Layer is implemented as Distributed, Per-Router Control

Historically, the Internet’s control plane was distributed:

- Each router contained:

  - Switching hardware (data plane)

  - Control software implementing routing protocols (IP, RIP, OSPF, IS-IS, BGP)

- Each vendor implemented proprietary router OS (e.g., Cisco IOS).

- Middleboxes (firewalls, load balancers, NATs, etc.) handled specialized network functions separately.

Drawbacks:

- Closed, vendor-specific systems.

- Complex management (misconfigurations common).

- Difficult traffic engineering and innovation.

2. Rethinking the Control Plane (~2005 Onwards)

Renewed interest in redefining the network control plane led to the idea of Software-Defined Networking (SDN).

3. Traditional Per-Router Control Plane

- Each router ran its own routing algorithm component.

- Routers interacted with each other to compute forwarding tables through distributed protocols.

- Limited global view → difficult to coordinate and optimize network-wide behavior.

4. SDN Control Plane Concept

- A remote (centralized) controller computes and installs forwarding tables in routers/switches.

- Switches perform data forwarding only.

Key Difference

| Traditional | SDN |
| --- | --- |
| Control plane distributed across routers | Control plane logically centralized in controller |
| Each router computes its table | Controller computes all tables and installs them |

5. Why a Logically Centralized Control Plane?

Advantages

- Simplified network management:

  - Avoid router misconfigurations.

  - Global view enables smarter decisions.

- Programmable forwarding:

  - Table-based forwarding (e.g., via OpenFlow API).

  - Centralized “programming” of routing tables.

- Open, non-proprietary implementation:

  - Promotes innovation and experimentation.

Analogy:
Similar to the mainframe → PC revolution:

| Traditional Networks | SDN |
| --- | --- |
| Vertically integrated (specialized hardware, OS, apps) | Horizontal layers with open interfaces |
| Proprietary, slow innovation | Rapid innovation, large open ecosystem |

6. Traffic Engineering Challenges in Traditional Networks

Example Questions:

- Force specific path:

  - How to make traffic from u → z follow u–v–w–z instead of u–x–y–z?

  - need to re-define link weights so traffic routing algorithm computes routes accordingly

  - Only control knob = link weights, making control hard.

- Split traffic for load balancing:

  - Split u → z traffic along two paths (u–v–w–z and u–x–y–z)? → Difficult.

- Different treatment for different flows:

  - If w wants to route “blue” and “red” traffic differently → Impossible with destination-based routing.

Conclusion:
Traditional routing (DV/LS) is limited.
→ SDN enables generalized flow-based forwarding to achieve any desired routing behavior.

- Key Features of SDN

8. SDN Architecture Overview

A. Data Plane (Switches)

- Fast, simple, commodity switches (hardware-based).

- Implement flow-table-based forwarding.

- Controlled via APIs (e.g., OpenFlow).

- Switch’s flow table:

  - Computed and installed by the controller.

  - Specifies match–action rules (e.g., forward, drop, modify).

B. Control Plane (SDN Controller)

- Acts as the Network Operating System (Network OS).

Responsibilities:

- Maintain global network state.

- Provide APIs:

  - Northbound API → For control applications above.

  - Southbound API → For switches below.

- Implemented as a distributed system for:

  - Scalability

  - Fault tolerance

  - Robustness

C. Network-Control Applications

- The “brains” of the SDN system.

- Use services from the controller (via APIs) to:

  - Implement routing, load balancing, access control, etc.

- Unbundled software: Can be written by third-party developers, independent of the controller or vendor.

9. Components of an SDN Controller

10. OpenFlow Protocol

Purpose

- Standardized protocol between controller and switch.

- Runs over TCP (optionally encrypted).

Three Classes of Messages

- Controller → Switch

- Asynchronous (Switch → Controller)

- Symmetric (Miscellaneous)

Note: OpenFlow protocol ≠ OpenFlow API
API specifies flow rules; protocol defines message exchange.

11. OpenFlow Message Types

Controller → Switch

| Message | Function |
| --- | --- |
| FEATURES | Query switch capabilities. |
| CONFIGURE | Set or get switch parameters. |
| MODIFY-STATE | Add/delete/modify flow entries in tables. |
| PACKET-OUT | Send specific packets out of selected ports. |

Switch → Controller

| Message | Function |
| --- | --- |
| PACKET-IN | Send packet to controller for decision. |
| FLOW-REMOVED | Notify controller when flow expires or is deleted. |
| PORT-STATUS | Inform controller about port state changes. |

12. Example: SDN Control/Data Plane Interaction

- S1, experiencing link failure uses OpenFlow port status message to notify controller

- SDN controller receives OpenFlow message, updates link status info

- Dijkstra’s routing algorithm application has previously registered to be called when ever link status changes. It is called.

- Dijkstra’s routing algorithm access network graph info, link state info in controller, computes new routes

- Link state routing app interacts with flow-table-computation component in SDN controller, which computes new flow tables needed

- Controller uses OpenFlow to install new tables in switches that need updating

13. ONOS SDN Controller

ONOS (Open Network Operating System)

A distributed, open-source SDN controller designed for scalability and reliability.

Features:

- Separation of network control apps from the core.

- Intent framework:

  - High-level “what” specification instead of “how”.

  - Example: “Connect host A to host B with 1 Gbps QoS.”

- Distributed core:

  - Provides service reliability, replication, and scalability.

Interfaces:

- Northbound: REST API (apps → controller)

- Southbound: OpenFlow, NetConf, OVSDB (controller → devices)

14. Google ORION SDN Control Plane

Overview

- ORION = Google’s SDN control plane (used in data centers (Jupiter) and WAN (B4)).

- Implements:

  - Intra-domain routing, iBGP.

  - Traffic engineering and CoFlow scheduling.

- Architecture:

  - Core built as distributed microservices (pub/sub).

  - Uses OpenFlow for switch signaling and monitoring.

Key Idea

Logical centralization simplifies configuration management.

Benefits (per Google’s engineers):

- All network element roles are pre-defined and centrally managed.

- Eliminates complex per-device configuration.

- Simplifies both technical implementation and human management.

15. SDN: Selected Challenges

| Challenge | Description |
| --- | --- |
| Control plane reliability | Must be dependable, secure, scalable, and fault-tolerant. |
| Failure robustness | Use reliable distributed systems theory. |
| Security and dependability | Should be built in “from day one”. |
| Mission-specific requirements | Real-time, ultra-reliable, ultra-secure networking (e.g., 5G). |
| Internet-scale deployment | Extending SDN beyond a single AS. |
| Integration with 5G | SDN plays a critical role in 5G core and slicing. |

16. SDN and Future of Traditional Protocols

| Comparison | Description |
| --- | --- |
| SDN-computed vs Router-computed forwarding | Centralized computation replaces distributed routing protocols. |
| Potential evolution | SDN-based congestion control – controller adjusts sender rates based on network feedback. |
| Big Question | How will implementation of network functionality evolve — centralized SDN or traditional distributed protocols? |

- ▪ Internet Control Message Protocol