Chapter 7 Wireless and Mobile Networks

-------------------------------------------------------------------------

- Wireless

- Wireless Links and network characteristics

Wireless Link Characteristics

1. Fading (Attenuation)

Wireless radio signal attenuates (loses power) as it propagates.

Free Space Path Loss

- Free space path loss ∼ (f d)²

2. Multipath Propagation

4. Wireless Link Characteristics: Noise

Noise Sources

- Interference from motors, appliances, and other sources using wireless frequencies.

SNR: Signal-to-Noise Ratio

- Larger SNR → easier to extract signal from noise (good).

SNR vs BER Tradeoff  (Bit Error rate)

- Given a physical layer:

  - Increase power → increase SNR → decrease BER.

- SNR may change with mobility →
Dynamically adapt physical layer:

  - Modulation technique

  - Data rate

5. Hidden Terminal Problem

- CDMA: code division multiple access

1. Basic Concepts

- A unique “code” is assigned to each user → code set partitioning.

- All users share the same frequency, but each user has its own “chipping sequence” (code) to encode data.

- This allows multiple users to coexist and transmit simultaneously with minimal interference, if codes are orthogonal.

2. CDMA Encoding and Decoding

Encoding

- Operation: Inner product

- Formula:
Encoded data = (original data) × (chipping sequence)

Decoding

- Operation: Summed inner-product

- Formula:
Decoded data = Σ (encoded data × chipping sequence)

- WiFi: 802.11 wireless LANs

802.11 LAN Architecture

802.11: Channels

  - Spectrum is divided into channels at different frequencies.

  - AP administrator chooses channel for the AP.

  - Interference possible:

  - Neighboring AP may choose the same channel.

802.11: Association

Arriving host must associate with an AP

Steps:

- Host scans channels, listening for beacon frames containing:

  - AP’s name (SSID)

  - AP’s MAC address

- Host selects an AP to associate with.

- Host may perform authentication (Chapter 8).

- Host typically runs DHCP to get IP address in AP’s subnet.

802.11: Passive vs Active Scanning

1. Passive Scanning

IEEE 802.11: Multiple Access

Goal: Avoid collisions

- Two or more nodes transmitting at the same time → collision.

Mechanism: CSMA (Carrier Sense Multiple Access)

- Sense channel before transmitting.

- Do not transmit if another transmission is detected.

Why no collision detection in 802.11?

- Hard to detect collisions because:

    - Transmitted signal is very strong.

    - Received signal is weak due to fading.

- Hidden terminal problem and fading prevent reliable collision detection.

Therefore: 802.11 uses CSMA/CA

- Collision Avoidance, not collision detection.

IEEE 802.11 MAC Protocol: CSMA/CA

Sender Behaviour

- If channel is idle for DIFS → transmit entire frame (no CD).

- If channel is busy, start random backoff timer:

  - Timer counts down only when the channel is idle.

  - Transmit when timer expires.

- If no ACK received:

  - Increase backoff interval.

  - Repeat step 2.

Receiver Behaviour

- If frame received OK:

  - Send ACK after SIFS.

  - ACK required due to hidden terminal problem.

Avoiding Collisions (RTS–CTS Mechanism)

Idea

Sender reserves channel before sending data.

Procedure

- Sender sends RTS (Request to Send) using CSMA.

  - RTS packets may collide, but are short.

- AP (base station) sends CTS (Clear to Send).

- CTS heard by all nodes:

  - Sender transmits DATA.

  - Other stations defer transmissions

.

802.11 Frame: Addressing

802.11: Mobility Within Same Subnet

- Host stays in same IP subnet → IP address stays same.

- Switch must determine which AP the host is currently associated with.

- Switch uses self-learning (Chapter 6):

  - Learns which switch port receives frames from H1.

802.11: Advanced Capabilities

1. Rate Adaptation

- Base station and mobile dynamically adjust transmission rate.

- Rate depends on SNR. Signal-to-Noise Ratio

- Behavior:

- As node moves away from AP:

  - SNR decreases

  - BER increases

- When BER becomes too high:

  - Switch to lower transmission rate

  - Lower rate → lower BER Bit Error Rate

2. Power Management

- Node tells AP: “I am going to sleep until next beacon frame.”

  - AP will not send frames to the sleeping node.

- Node wakes before next beacon frame.

- Beacon frame contains list of mobiles with AP-to-mobile frames waiting.

  - If frames are waiting → node stays awake.

  - Otherwise → node sleeps again until next beacon.

- Bluetooth

Personal Area Networks: Bluetooth

Basic Characteristics

- Coverage: Less than 10 m diameter

- Purpose: Replacement for cable

- Ad hoc network: no infrastructure

- Operating band: 2.4–2.5 GHz ISM band

- Data rate: Up to 3 Mbps

Master–Client Architecture

- Cellular networks: 4G and 5G

4G / 5G Cellular Networks — Complete Structured Notes

1. Overview

- 4G/5G is the solution for wide-area mobile Internet.

- Widespread deployment:

- More mobile-broadband devices than fixed broadband (5:1 in 2019).

- 4G availability: 97% in Korea, 90% in the US.

- Transmission rates: up to hundreds of Mbps.

- Technical standards by 3GPP (3rd Generation Partnership Project)

- 4G standard: LTE (Long-Term Evolution)

2. 4G/5G: Similarities vs Differences with Wired Internet

Similarities

- Edge/core distinction (both operated by same carrier).

- Global cellular network = a network of networks.

- Uses familiar protocols:

  - HTTP, DNS, TCP, UDP, IP, NAT

  - Data/control plane separation

  - SDN

  - Ethernet

  - Tunneling

- Interconnected with wired Internet.

Differences

- Wireless link layer is different.

- Mobility is a first-class service.

- User identity via SIM card.

- Subscription-based business model:

  - Strong notion of home network and visited/roaming networks.

  - Global access with authentication infrastructure and inter-carrier settlements.

3. Elements of 4G LTE Architecture

3.1 User Equipment (UE)

- Smartphones, tablets, laptops, IoT devices with LTE radio.

- Has 64-bit IMSI (International Mobile Subscriber Identity) stored on SIM card.

3.2 Base Station (eNode-B)

- At the edge of the carrier’s network.

- Manages radio resources and connected mobile devices.

- Coordinates authentication with MME and HSS.

- Compared with WiFi AP:

  - Plays active role in user mobility.

  - Coordinates with nearby base stations to optimize radio use.

3.3 Radio Access Network (4G Radio)

- Connects UE to eNode-B.

- Many possible frequency bands:

  - Popular bands: 600, 700, 850, 1500, 1700, 1900, 2100, 2600, 3500 MHz

- Separate uplink and downlink channels.

- Channel sharing among users:

  - OFDM (Orthogonal Frequency Division Multiplexing)
→ combination of FDM + TDM.

- Up to hundreds of Mbps per device.

3.4 Home Subscriber Service (HSS)

- Stores information for devices whose home network this is.

- Works with MME for authentication.

3.5 Serving Gateway (S-GW) & PDN Gateway (P-GW)

Serving Gateway (S-GW)

- Routes traffic between eNode-B and P-GW.

- Supports mobility with extensive tunneling.

PDN Gateway (P-GW)

- Gateway to the mobile cellular network.

- Appears like a regular Internet gateway router.

- Provides NAT services.

3.6 Mobility Management Entity (MME)

- Handles device authentication (device-to-network & network-to-device) with HSS.

- Mobile device management:

  - Handover between cells.

  - Tracking device location.

  - Paging device.

- Sets up tunneling path from UE to P-GW.

- 4G LTE: Data Plane & Control Plane Separation

5. LTE Data Plane Protocol Stack

6. LTE Data Plane: Association with a Base Station

- BS broadcasts primary sync signal every 5 ms on all frequencies.

  - Multiple carriers may broadcast sync signals.

- Mobile detects:

  - Primary sync signal

  - Secondary sync signal

  - BS information (bandwidth, configuration, carrier info)

- Mobile may detect multiple base stations/networks.

- Mobile selects BS to associate with (prefers home carrier).

- Further authentication, state establishment, data-plane setup occurs.

7. LTE Mobile Sleep Modes

- Light sleep: after hundreds of milliseconds of no activity.

- Wakes periodically to check for downstream data.

- Deep sleep: after 5–10 seconds of inactivity.

- UE may change cells while sleeping → must re-establish association.

8. Global Cellular Network: A Network of IP Networks

- Mobility

- Mobility management: principles

Mobility — Complete Notes

1. What is Mobility?

Mobility refers to how a device moves across networks from the network's perspective.

2. Mobility Challenge

When a device moves from one network to another:How will the network know where to forward packets?

3. Mobility Approaches

Approach 1: Let the Network (Routers) Handle Mobility

- Routers advertise the well-known name/address (e.g., permanent IP address or location number) of visiting mobile nodes.

- Achieved through routing table exchanges.

- Internet routing can do this already using longest prefix match.

Problem

❌ Not scalable to billions of mobile devices.

Approach 2: Let End-Systems Handle Mobility (Edge Functionality)

Two Methods

- Indirect Routing

  - Correspondent → Home network → Mobile

  - Home network forwards packets to mobile’s current location.

- Direct Routing

  - Correspondent learns mobile’s foreign address.

  - Correspondent sends datagrams directly to mobile.

4. Contacting a Mobile Friend (Analogy)

If a friend frequently changes location, how do you find them?

Options:

- Search all phone books?

- Expect them to notify you?

- Call their parents?

- Facebook! → A definitive “home” where you can find their current location.

Importance of a “Home”

- A place that always knows where you are.

5. Home Network & Visited Network — 4G/5G

6. Home Network & Visited Network — ISP/WiFi

7. Home & Visited Networks — Generic Model

8. Registration Process

9. Mobility with Indirect Routing

Indirect Routing: Comments

Triangle Routing Problem

- Inefficient when correspondent & mobile are in the same network.

Mobility Support

- Mobile moves among visited networks transparently:

  - Registers in new visited network

  - New visited network registers with home HSS

  - Home network continues forwarding datagrams

  - Ongoing connections (e.g., TCP) do not break

10. Mobility with Direct Routing

Process

- Correspondent contacts home HSS

- Home HSS provides mobile’s current visited network

- Correspondent addresses datagrams to that visited network

- Visited gateway forwards to mobile

Direct Routing: Comments

- Benefits

  - Fixes triangle routing inefficiency.

- Downsides

  - Not transparent to correspondent:

  - Correspondent must obtain care-of address.

  - If mobile changes network:

  - Additional complexity required.

- Mobility management: practice

  - 4G/5G networks

  - Mobile IP

- Mobility: impact on higher-layer protocols

Mobility in 4G Networks — Major Mobility Tasks

Configuring LTE Control-Plane Elements

Steps

- Mobile communicates with local MME via base-station control-channel.

- MME uses mobile’s IMSI to contact home HSS:

  - Retrieve authentication, encryption, network service information.

  - Home HSS learns mobile is now in visited network.

- Base station & mobile select radio-channel parameters for data-plane communication.

Configuring Data-Plane Tunnels for Mobile

- S-GW ↔ Base Station Tunnel

- When mobile changes base stations:

  - Only change endpoint IP address of the tunnel.

- S-GW ↔ Home P-GW Tunnel

- Implements indirect routing.

- Heavy use of GTP (GPRS Tunneling Protocol):

  - Mobile’s datagram → encapsulated in GTP → UDP → IP.

Handover Between Base Stations (Same Provider Network)

Mobile IP

- Standardized ~20 years ago (RFC 5944), but never widely deployed.

- At that time, WiFi + 2G/3G was “good enough,” so full Mobile IP wasn’t needed.

- Uses indirect routing via tunneling through the home network.

- Uses ICMP-based protocols for:

  - discovering foreign agents

  - registering new location with home agent

Wireless/Mobility → Impact on Higher Layers

- In theory: higher layers (TCP/UDP) should work normally because the service model is the same.

- In practice: performance suffers due to:

  - wireless bit errors

  - link-layer retransmissions

  - handover losses

- TCP wrongly interprets wireless losses as congestion → reduces window unnecessarily.

- Real-time traffic (voice/video) suffers from added delay.

- Wireless bandwidth is limited → makes impairments worse.